To: Jay Hardy and Jordan Swisher

From: Josh Radoff, Founder Arroyo Climate Advisers and CU Professor Renewable and Sustainable Energy

Date: June 2, 2021

Subject: Redtail Ridge Sustainability Commitment Plan

## Overview

The Redtail Ridge site, formerly the Conoco Phillips property, is a parcel of land adjacent to US 36 that was previously demolished and then left abandoned by the previous developer. From a sustainability standpoint there are several opportunities to implement best practices that support decarbonization, improve habitat and water quality, improve community trail access and wellbeing, and create beneficial land use. The following areas summarize the development’s efforts and exploration in the various respective impact areas.

To establish a foundation of sustainable development, all office and retail buildings larger than 10,000 square feet will be required to achieve a minimum of LEED BD+C Silver Certification (using the most current version of the rating system available, currently v4.0 with v4.1 supplements) through the Core and Shell or New Construction programs. Additionally, all industrial buildings will be required to be LEED BD+C Certified (using the most current version of the rating system available, currently v4.0 with v4.1 supplements) through the Core and Shell or New Construction programs. To facilitate this, the master plan will pursue a campus approach to the Location and Site credits, meaning that each of the vertical developments can focus only on the areas within their scope. Within this, the program will require that each project achieve at least one construction waste management point, one outdoor water efficiency point, one indoor water efficiency point, and one optimize energy performance point to demonstrate sustainability balance. The project will also pursue points in the priority areas of daylight, air quality, and renewable energy.

| Summary |  |
| --- | --- |
| Commitment | LEED v4.0 BD+C (CS or NC) Silver will be required for all office and retail buildings greater than 10,000 square feet and LEED v4.0 BD+C (CS or NC) Certified will be required for all industrial buildings |
| Commitment | At least one point will be required to be earned in the following credit areas: construction waste management, outdoor water efficiency, indoor water efficiency, and optimize energy performance |

## Transportation and Mobility

- There are major impacts associated with transportation and mobility are greenhouse gas (GHG) emissions, regional air quality, land use, and health and wellbeing.  To address these areas the project will carry out the following. Please see the full Transportation Demand Management (TDM) plan for a more detailed list of options to reduce single occupancy vehicle use to and from the site.

Trail access and bike lanes. Given that the property sits on the US 36 bike corridor and Coal Creek Trail network, the site is highly accessible by bike from across the region. The site can also be used to connect to the Marshall Mesa recreational trail network to the west of US 36. To further support walking and biking, the interior street networks will include complete street design, allocating full bike lanes and sidewalks to enable walking and biking across and throughout the site.  These items will be built in the initial phase.

Beyond the TDM commitments for long-term bike facilities and amenities, the development will provide shared electric bikes for use by tenants and will contract with local food truck vendors to provide easily accessible food options and minimize lunch trips during work hours.

Public transit. The site can be accessed by several RTD bus lines within a mile to a mile and a half walking distance. To support the last mile connectivity, the project will fund a circulator shuttle to bring passengers from the Flatirons station along US 36.

EV Charging Infrastructure. The site will provide infrastructure to support EV charging. This will include making all parking lots and structured parking EV-ready per the LEED v4.1 Electrical Vehicle credit , as well as providing a series of Level 2 charging stations in partnership with a charging management vendor. If the site is considered able and appropriate to support Level 3 Fast Charging, the project will allocate space to enable its inclusion.

Parking. The project will not provide more parking than the City’s minimum requirements. Structured parking to be incentivized.

| Summary |  |
| --- | --- |
| Commitment | Complete street design as outlined in the City of Louisville’s planning department |
| Commitment | All parking areas will be constructed as EV ready. In addition, 3% of parking spaces provided will have level 2 chargers |
| Commitment | The project will not exceed the City’s minimum requirements for parking spaces. |
| Commitment | Funding for a circulator shuttle to provide service between transit stops and the site |
| Commitment | Shared electric bike fleet available for tenants |
| Commitment | Food truck vendors to be coordinated to provided regular lunch service at the site |
| Goal | Structured parking facilities for a portion of the site parking |

## Site Development and WAter

The overall site will be developed with the goal of preserving as many of the high value habitat and natural areas, especially on the northern edge of the property, while enhancing and creating new areas and corridors throughout the site.  The landscape plan focuses on the use of native vegetation, creating pollinator pathways, and taking the beginnings of those habitat areas that have begun to develop around the previous stormwater detention ponds and embellishing those to a healthier, more biodiverse landscape. In addition, the project will implement the following measures.

- Implement a habitat management plan for the full site (400+ acres) in partnership with High Plains Environmental Center.

- Provide conservation easements (~40 acres) on the southeastern portion of the site

Limit the use of grass landscaping and instead feature native landscaping elements with low watering and mowing requirements. Native and water-wise landscaping elements will be prioritized.

As raw water rights are available, these will be used for the limited irrigation demands of the site.

Create a “biophilic” design throughout, allowing occupants and visitors the ability to connect with nature, such as through walking paths, gardens, and natural areas.

Develop a site-wide green infrastructure plan, managing and infiltrating stormwater (especially from roofs and surface parking) via natural elements such as rain gardens, swales, etc.

To support interior water efficiency, the project will require a 30% reduction from the LEED v4.0 baseline on water consumption fixtures and fittings.

| Summary |  |
| --- | --- |
| Commitment | Implement a site-wide habitat management plan developed by a qualified habitat professional |
| Commitment | 39.7 acres of dedicated open space will be provided to the City of Louisville |
| Commitment | Native and water-wise landscaping elements will be prioritized with turf only being provided if deemed necessary. |
| Commitment | Biophilic oriented site plan |
| Commitment | Development of a green infrastructure plan to manage stormwater |
| Commitment | All buildings will be required to use low water indoor consumption fixtures and fittings 30% below the LEED v4.0 baseline |
| Commitment | The site will demonstrate a 50% reduction in outdoor potable water use below the LEED v4.0 baseline. |
| Goal | As raw water rights are available, these will be used for the limited irrigation demands of the site. |

## EnergY and CarBon

The clean energy landscape has evolved considerably over the past several years to address the importance of mitigating climate change. Notably, Xcel Energy (which will serve the site) has committed to an 80% reduction in GHG emissions by 2035 and a 100% reduction target by 2050. This project will implement several best practices to move towards building decarbonization, including the following areas and measures. We will also work closely with Xcel to take advantage of all relevant programs to support the project goals, including applying for their Partners in Energy Program.

- Energy Efficiency. The current City of Louisville code requires that all projects meet IECC 2018, and it will update to IECC 2021 sometime in the near future. This will require all development to achieve a significant amount of energy effiency by meeting the code baseline. Given that all buildings greater than 10,000 square feet will be required to be LEED Certified, this will drive additional energy efficiency.  In addition, all tenants will be separately sub-metered and charged for electricity consumption, even if Xcel meters for the project at the building level.

Building Electrification. From a GHG reduction perspective, one of the most important measures will be to convert all combustion based HVAC equipment to electricity. The project will require all office buildings to use electric-based HVAC systems.

Solar PV. Given the availability of roof area, this will be one of the biggest single opportunities for carbon reduction at the site. Both rooftop and parking canopy solar PV will be evaluated. The project will develop a minumum of 2 MW of onsite solar PV in proportion with the project buildout. In addition, all buildings will be required to be solar ready, meaning they will allocate at least 50% of their roof area for solar, include the required structural load for ballasted solar arrays, and include conduit to support future installation.

Heat Island Effect: White roofs will be be required for all buildings.

Depending on the level of onsite renewable energy development that can be achieved, the project may subscribe to offsite renewable energy projects such as local community solar garden (CSG) developments.

Low embodied carbon materials, including those that are locally manufactured and use recycled content will be prioritized.

Net Zero Energy. By implementing all of the strategies above (efficiency, electriciation, onsite solar, offsite renewable procurement), one or more buildings may be eligible to be designated as Net Zero Energy (NZE). Therefore, these projects will coordinate with the USGBC and International Living Future Institute (ILFI) to better understand how each organization’s defition of NZE could be applied.

| Summary |  |
| --- | --- |
| Commitment | All buildings will meet IECC 2021 |
| Commitment | All tenants will be separately sub-metered and charged for electricity consumption |
| Commitment | All buildings will be solar ready |
| Commitment | Project to develop and install a minimum of 2 megawatts of solar capacity on site, in proportion with building area coming online |
| Commitment | All office buildings to utilize electricity-only HVAC systems |
| Commitment | Participate in Xcel programs to support efficiency and renewable energy, such as Energy Design Assistance, Solar Rewards, and Partners in Energy |
| Goal | All buildings will install solar on >50% of their roof area |
| Goal | Procure 5 - 100% of the electricity demand from a CO-based community solar garden. |
| Goal | Certify one or more buildings to net zero energy using LEED Zero or ILFI’s Net Zero Energy program. |
| Goal | Recycled and locally produced concrete and other materials to be specified |

## Health

One of the primary goals of the site is to create a healthy commercial workplace for local residents. The landscape plan and bike and walking aspects of the mobility plan are some of the primary means to allow people to be active and to connect with nature in and around the site. With regard to the vertical development of the buildings, the following will be implemented as part of the project’s design:

- Walkable stairs. At least one staircase in all buildings will be designed to encourage walking to all floors throughout the building.

- Exercise facilities. The site will consist of at least one exercise facility as an amenity for tenants.

- The project will conduct periodic indoor air quality (IAQ) monitoring and testing to proactively identify any issues that arise with ventilation systems or chemicals introduced to building air flow.

- All office buildings will be encouraged to pursue the LEED daylight credit instituting a minimum level of spatial daylight autonomy (sDA).

- All buildings will be required to include some amount of operable windows to enable air flow within selected portions of each building.

- Included in the landscape plan will be a series of shaded, outdoor spaces with seating for each sub block of the master plan.

- Pandemic design best practices. The project design team will review the lessons learned and recently developed standards to enable buildings to be safer places in the event of a future pandemic.

| Summary |  |
| --- | --- |
| Commitment | All multi-story buildings to have on active design stairway |
| Commitment | Shared, outdoor landscape areas with seating to be included |
| Commitment | Pandemic design best practices to be included in HVAC design |
| Commitment | At least one exercise facility will be included on the site |
| Commitment | Annual indoor air quality testing to be performed |
| Commitment | All office buildings and office buildouts of industrial or R&D buildings to include operable windows |
| Goal | All office buildings to pursue the LEED daylight credit |

## Operations

Ensuring the ongoing performance of the buildings and implementation of the various strategies that define sustainability for the site is a critical element of ensuring long term project success. Each of the areas above will have a corresponding operations and maintenance program, whether landscaping, transportation demand, energy, or indoor air quality, and specific plans will be developed for each in turn as the site design and program evolves. The following are aspects not otherwise addressed above that the development is committed to instituting for the project:

- Site-wide waste management. The development will evaluate existing services to provide composting and recycling (including e-waste) for all of the buildings and tenants. This will effectively enable any tenant to pursue zero waste goals.

- Tenant sustainability program. The development will facilitate a sustainability business program among tenants, outlining various measures that are high priorities for office and manufacturing tenants, including procurement, waste diversion and e-waste management, transportation incentives, etc.

- Education. The development will facilitate various educational opportunities for tenants, and will explore partnering with the adjacent middle and high school for student environmental education and engagement on the site.

| Summary |  |
| --- | --- |
| Commitment | Composting and recycling service provided to all buildings |
| Commitment | Sustainability education about the site features to be provided |
| Commitment | Tenants will participate in the County’s Partners for a Clean Environment program |